# GPON Fault PoC: Knowledge Transfer Guide

Oct 20, 2026 · @Satya

This guide takes a developer who is new to telecom, fault management and this codebase from zero to productive. After one read you can run the system, explain every part, follow one fault from the network to the screen, and make common changes safely. The same text ships in the zip as docs/KNOWLEDGE_TRANSFER.md.

## 1. What this project is, in one page

Telecom operators run large networks that constantly produce alarms: messages from equipment saying "something is wrong here". One physical fault (say, a fibre cut by a digger) causes dozens of alarms from different devices, because everything downstream of the cut also fails. A human operator in a Network Operations Centre (NOC) has to work out which alarms belong together, what really broke, who is affected, and what to do. That is slow: in our baseline history the average time to resolve is about two hours.

This project is a proof of concept (PoC) that automates that work for one kind of network: a mobile backhaul built from GPON fibre access and G.702 E1/E3 TDM circuits. It:

- Ingests alarms and telemetry in the formats real equipment uses (SNMP traps, syslog, event streams, performance counters).

- Correlates related alarms into a single incident using the network topology and timing.

- Finds the root cause with a probabilistic (Bayesian) model, even when the broken thing cannot report (a fibre).

- Assesses impact: which services, cell sites and how many subscribers are affected.

- Decides and acts in a closed loop (detect, analyze, decide, act, verify), within configurable autonomy limits.

- Verifies the fix against fresh telemetry and escalates to a human when the fix fails or is too risky.

- Survives its own failures: alarm storms, duplicate or out-of-order messages, a crashed correlation engine, the AI being down, unreachable devices.

- Shows everything on a web dashboard with KPIs such as MTTD and MTTR.

The network itself is simulated, so the demo runs on a laptop. The important design rule is that the simulator only talks to the rest of the system through raw messages, exactly as real equipment would. Everything after ingestion is "real" logic that does not know it is talking to a simulator.

Tech stack: Python 3.10+ (FastAPI, asyncio, NumPy, scikit-learn, PyYAML, SQLite) for the backend; React 18 + Vite + Recharts for the dashboard. No Docker: a Python virtual environment and npm.

## 2. Telecom basics you need

You do not need to be a network engineer, but you do need these concepts.

### 2.1 Mobile backhaul

A mobile phone talks over radio to a cell site (a base station). The cell site must then be connected to the operator's core network. That connection is called backhaul. In our model there are two generations of cell sites:

- 2G sites (BTS) speak old-style TDM (time-division multiplexing) over E1 circuits to a controller called the BSC (Base Station Controller). The link between BTS and BSC is called Abis.

- 4G sites (eNB) and enterprise customers use ordinary Ethernet/IP.

### 2.2 GPON (the fibre access network)

GPON (Gigabit Passive Optical Network, ITU-T G.984) brings fibre to many endpoints by splitting light passively:

OLT  --PON port--feeder fibre--> SPLITTER (1:8) --drop fibre--> ONT  (at the cell site / customer)
(in the exchange)                (in a street cabinet)                (box at the site)

| Term | Meaning |
| --- | --- |
| OLT (Optical Line Terminal) | The active box in the operator's building. Has many PON ports. |
| PON port | One optical port on the OLT; one fibre tree hangs off it. |
| Feeder fibre | Fibre from the PON port to the splitter. Cutting it kills everything behind the splitter. |
| Splitter | Passive glass device that divides light 1 to 8 (or 2 to 8). Has no power and cannot send alarms. |
| Drop fibre | Fibre from the splitter to one ONT. |
| ONT (Optical Network Terminal) | Box at the site. Cell-site ONTs also have E1 ports (see below). |
| ODN | Optical Distribution Network: all the passive fibre and splitters between OLT and ONTs. |
| Type-B protection | A splitter fed by two feeders from two PON ports; if the working feeder breaks, the OLT switches to the standby feeder. |

Key GPON alarms (from G.984.3) you will see: PON_LOS (the port receives no light), ONT_LOSi (the OLT lost contact with ONT i), ONT_SDi (signal degraded for ONT i), PON_BER_SD (bit-error rate too high on the port), ONT_DYING_GASP (the ONT lost power and sent a last message).

Optical receive power is measured in dBm (negative numbers; -18 dBm is healthy, below -27 dBm the ONT can no longer decode the signal for "class B+" optics). Our prediction model watches this number drift down.

### 2.3 G.702 and E1 (the TDM side)

ITU-T G.702 defines the classic digital hierarchy bit rates. The two we use:

- E1 = 2048 kbit/s: the basic circuit carrying 2G Abis traffic (framing from G.704).

- E3 = 34368 kbit/s: carries 16 E1s, produced by a PDH multiplexer (our G702Mux).

Because the GPON network is packet-based, E1 circuits are carried across it using circuit emulation (CES): the ONT turns the E1 into packets, a pseudowire (CESoPSN, RFC 5086) carries them across the network, and a CES gateway in the core turns them back into E1s and hands them to the BSC over a channelized STM-1 link.

Key G.702-side alarms: E1_LOS (no signal on the E1), E1_AIS (an "all ones" signal meaning "something upstream failed"), E1_FRAME_SLIP (clock mismatch, G.822), E1_ERRORED_SECONDS (bit errors, G.826), PW_DOWN (pseudowire down), PW_JITTER_BUFFER_UNDERRUN (packets arrive too irregularly), ABIS_LINK_DOWN (BSC lost the cell).

### 2.4 Layers

- access: OLT, PON ports, fibres, splitters, ONTs, BTS, E1 circuits

- aggregation: aggregation switches, OLT uplinks, the PDH mux and its E3 trunk

- core: core routers, the CES gateway, the BSC, pseudowires, STM-1

- service: the end-to-end products (2G Abis backhaul, 4G backhaul, enterprise L2VPN)

### 2.5 Why one fault makes many alarms

If feeder fibre F-A1 is cut:

- the OLT sees no light on PON-A1, so it raises PON_LOS (SNMP trap from OLT-A)

- the OLT loses four ONTs: 4 x ONT_LOSi

- two of those ONTs carried E1s, so the CES gateway reports 2 x PW_DOWN and 2 x E1_AIS (syslog)

- the BSC loses two cells: 2 x ABIS_LINK_DOWN (event stream)

- service assurance sees 4G and enterprise services down: 3 x SERVICE_SLA_BREACH

That is about 14 to 15 alarms, from three different systems, in three different formats, and none of them is on the fibre, because a fibre cannot report. Turning those into "one incident, root cause F-A1, 5 services and 2 cell sites affected, switch to the standby feeder" is the core job of this system.

## 3. Fault-management basics

| Concept | Meaning in this project |
| --- | --- |
| Alarm | One normalised fault message: type, severity, entity, reporter, timestamps, state (active or cleared). |
| Raise / clear | Devices send a "raise" when a condition starts and a "clear" when it ends. |
| Severity | critical > major > minor > warning > info. |
| Category | link_down, signal_degradation, timing_sync, equipment, configuration, performance. |
| Incident | A group of alarms believed to have one cause. The unit the closed loop works on. |
| Correlation | Deciding which incident a new alarm belongs to. |
| Compression | Raw alarms divided by incidents. Higher is better (less noise for humans). |
| RCA | Root-cause analysis: which entity actually failed. |
| Impact / blast radius | Everything downstream of the root cause; we count services. |
| Enrichment | Adding context to an alarm: where it is in the topology, device metadata, history. |
| Playbook | A scripted remediation (one or more steps) for a kind of fault. |
| Closed loop | The system acts on its own analysis and then checks whether it worked. |
| Autonomy level | advisory (only recommend), supervised (prepare, wait for approval), autonomous (just do it). |
| Guardrail | A limit that stops autonomous action (too risky, too many services, low confidence, too frequent). |
| Escalation | Handing an incident to a human with a ticket and full context. |
| MTTD | Mean Time To Detect: fault start to incident created. |
| MTTR | Mean Time To Resolve: fault start to service restored. |

## 4. Set up and run it (step by step)

### 4.1 Prerequisites

- Python 3.10 or newer (python3 --version). Tested on 3.13.

- Node.js 18 or newer with npm (node --version). Tested on Node 22.

- Internet access the first time, to install Python and npm packages.

- Ports 8000 (backend), 5173 (dashboard) and optionally 5514/UDP (syslog listener) free.

### 4.2 Unzip

unzip gpon-fault-poc.zip
cd gpon-fault-poc

### 4.3 Start the backend (terminal 1)

./run_backend.sh

What the script does (read run_backend.sh, it is short):

- cd backend

- If backend/.venv does not exist: python3 -m venv .venv and pip install -r requirements.txt

- uvicorn app.main:app --host 0.0.0.0 --port 8000

On start-up the backend seeds backend/data/history.db (SQLite) with 30 days of synthetic history (first time only), trains the anomaly and prediction models (about 1 second), and starts all background loops (simulator, ingestion, correlation engines, closed loop).

Check it: open http://localhost:8000/api/health. You should see {"status":"ok", ...}. Interactive API docs are at http://localhost:8000/docs. On Windows use .\run_backend.ps1 in PowerShell.

### 4.4 Start the dashboard (terminal 2)

./run_frontend.sh

This runs npm install the first time, then npm run dev (Vite on port 5173). Vite proxies every /api/... request to http://localhost:8000 (see frontend/vite.config.js). Open http://localhost:5173. On Windows use .\run_frontend.ps1; to start both at once use ./run_all.sh.

### 4.5 Optional: single port

cd frontend && npm run build      # creates frontend/dist
# restart the backend; it now also serves the built dashboard at http://localhost:8000

### 4.6 Reset to a clean state

- In the UI: Repair network (removes injected faults) or Reset all (also clears incidents).

- Fully fresh history: stop the backend, delete backend/data/history.db*, start again.

## 5. A guided tour of the dashboard

Header chips show the health of the platform itself: which correlation engine is active, whether the AI is online, ingestion state and queue depth. Banners appear for safe mode, failover and a lost backend connection.

KPI strip: MTTD, MTTR (with the rule-based baseline), alarm compression, % resolved autonomously, open incidents, waiting approvals, escalations, predicted faults.

Scenario bar: buttons that inject faults into the simulator. Hover a button to read its story. Guided demo runs four scenarios plus an engine crash. show simulator ground truth marks where faults were really injected, which is useful to check the AI.

Tab 1, Operations

- Topology: rows are Core, Aggregation, OLT, PON port, Splitter, ONT, Cell site. Colours: blue = GPON, purple = G.702, grey = IP. Alarmed nodes are filled with their severity colour; a pulsing red ring marks the AI's root cause; "RC" on a link marks an inferred passive fault (e.g. a fibre); a dashed feeder is the standby. Click anything for its ontology record.

- Correlated incidents: open first, then recently resolved. Click one to open the incident drawer.

- Real-time alarm feed: newest first; click a row to open its incident.

- System activity: platform events (scenarios, drills, actions, escalations, failovers).

Incident drawer: phase stepper (Detect, Analyze, Decide, Act, Verify, Resolved or Escalated); operator buttons (Approve, Reject, Execute, Acknowledge, Resolve); metrics; RCA hypotheses with confidence bars and evidence; impact; the policy decision and why; ranked playbooks with historical success; actions with verification checks; correlated alarms (click one for its enrichment); full timeline.

Tab 2, Closed loop: a board with four columns (awaiting approval, advisory, executing/verifying/watching, escalated) and the action log.

Tab 3, Autonomy & resilience: the editable autonomy matrix and guardrails; fault-tolerance drill buttons; ingestion pipeline gauges and counters; correlation engine instances and failovers; AI component status with active anomalies and predictions; circuit breakers; dead-letter queue.

Tab 4, Trends & KPIs: comparison table (rule-based history vs AI history vs this session), live charts (ingest vs alarm rate, queue depth, active alarms vs open incidents) and 30-day history charts.

Try this first: click Cascading fibre cut, wait 5 seconds, click the incident, read the RCA section, press Approve, and watch the stepper move to Resolved.

## 6. Repository map

gpon-fault-poc/
├── README.md                    quick start, demo script, API summary
├── run_backend.sh / .ps1        create venv + start backend
├── run_frontend.sh / .ps1       npm install + start dashboard
├── run_all.sh                   both
├── backend/
│   ├── requirements.txt
│   ├── config/                  ALL behaviour knobs (YAML), see section 10
│   │   ├── topology.yaml        ontology + every network element
│   │   ├── alarm_catalog.yaml   alarm types, categories, RCA evidence weights
│   │   ├── playbooks.yaml       remediation playbooks
│   │   ├── policy.yaml          autonomy matrix + guardrails
│   │   └── settings.yaml        timings, queue sizes, thresholds
│   ├── app/
│   │   ├── main.py              FastAPI routes (thin layer)
│   │   ├── runtime.py           builds everything, background loops, scenarios, drills, KPIs
│   │   ├── ontology.py          Topology graph + AlarmCatalog
│   │   ├── simulator.py         the fake network
│   │   ├── sources.py           wire formats + retrying senders
│   │   ├── ingestion.py         gateway, de-dup, re-order, event log
│   │   ├── correlation.py       correlation engine + active/standby supervisor
│   │   ├── enrichment.py        alarm enrichment
│   │   ├── ai.py                ML models, RCA, ranking, rule-based fallback
│   │   ├── closed_loop.py       policy, device adapter, closed-loop controller
│   │   └── history.py           SQLite: history, learned stats, checkpoints
│   ├── scripts/verify_demo.py   end-to-end test against a running backend
│   └── data/                    history.db (created at runtime)
├── frontend/
│   ├── package.json, vite.config.js, index.html
│   └── src/
│       ├── main.jsx, App.jsx    entry + page layout, tabs, polling
│       ├── api.js               fetch helper, usePoll hook, formatters
│       ├── styles.css           dark NOC theme
│       └── components/          TopologyView, IncidentDetail, Panels, ClosedLoopBoard, AutonomyResilience, Trends
└── docs/                        ARCHITECTURE, TOPOLOGY_ONTOLOGY, AI_LOGIC, CLOSED_LOOP, FAULT_TOLERANCE, METRICS, KNOWLEDGE_TRANSFER, screenshots

Rule of thumb: behaviour lives in YAML, mechanics live in Python, presentation lives in React. Most changes a product person asks for are YAML edits.

## 7. The architecture in one picture

┌──────────── simulator.py ────────────┐
 │ faults → status propagation → alarms │  + telemetry every 2 s, + background noise
 └───────────────┬──────────────────────┘
                 │ raw messages (SNMP trap dict / syslog line / JSON event / telemetry batch)
                 ▼
 sources.py   SourceClient x4  (buffer, delay jitter, retry with exponential backoff + full jitter)
                 ▼  submit()
 ingestion.py IngestionGateway: parse → validate (DLQ) → admission control (bounded queue, watermark shedding)
                 ▼  run() loop, 450 ev/s
              de-dup by event id → re-order buffer (1 s, by device time) → EventLog (offset-addressed)
                 ▼  read(offset)
 correlation.py  EngineSupervisor ── leader CorrelationEngine (corr-a)   standby (corr-b)
                 │                    alarm lifecycle, enrichment, correlation, merge
                 │                    telemetry → ai.analyze_telemetry → AI_ANOMALY / PREDICTED_FAULT alarms
                 │                    checkpoint state+offset to SQLite every 1 s
                 ▼  incidents (dicts inside the leader's state)
 closed_loop.py  ClosedLoopController.tick() every 0.5 s:
                 detected → analyze (ai.rca, impact, playbooks) → decide (policy+guardrails)
                 → act (DeviceAdapter → simulator.apply_action) → verify → resolved / escalated
                 ▼
 runtime.py   Runtime.snapshot()  ──►  main.py  GET /api/snapshot  ──► React dashboard (polls every 1 s)

Everything runs in one Python process on one asyncio event loop. Each box is written so it could become its own service (see docs/ARCHITECTURE.md, section "From PoC process to deployed services").

### Background tasks started by Runtime.start() (runtime.py)

| Task | Period | Job |
| --- | --- | --- |
| _sim_loop | 0.5 s | sim.tick(), render alarm events, hand them to sources (with occasional SNMP retransmits) |
| _telemetry_loop | 2 s | sim.telemetry() to the telemetry source |
| _noise_loop | 12 to 25 s | random realistic transients |
| _sampler | 2 s | records points for the live charts |
| gateway.run | 50 ms | drains the ingestion queue into the event log |
| supervisor.run | 0.25 s | leader processes the log, heartbeats, checkpoints; failover logic |
| loop.run | 0.5 s | closed-loop controller tick |
| SourceClient.run x4 | continuous | deliver buffered messages with retry |
| UDP endpoint | on packet | real syslog on :5514 |

## 8. Follow one fault end to end through the code

Scenario: you click Cascading fibre cut. Follow along in the files.

Step 1: the button. frontend/src/App.jsx calls run('/scenarios/cascade_fiber_cut'), which sends POST /api/scenarios/cascade_fiber_cut to backend/app/main.py (run_scenario), which calls Runtime.run_scenario in runtime.py, which calls sim.inject("F-A1", "cut").

Step 2: the simulator notices (simulator.py). Within 0.5 s _sim_loop calls NetworkSim.tick():

- _recompute() evaluates each entity's status: an entity is down if it has its own "down" fault or any depends_on parent is down. F-A1 is down, so SPL-A1, the drops, ONTs, E1s, BTS, pseudowires and services are all down.

- _expected_alarms() turns statuses into the alarms each element would report, for example the PON-port rule: "port is up but its active feeder is down, so the OLT reports PON_LOS with laser_tx=ok". Each alarm key is (reporter, entity, type).

- tick() diffs expected vs previously active keys and produces raise events (and clear events for keys that went away).

Step 3: wire formats (sources.py, render). The reporter class decides the format: OLT gives an SNMP trap dict (trap OID + varbinds); CES gateway, mux and IP devices give an RFC 5424 syslog line; BSC, NMS and SA give a JSON stream event. SourceClient.send buffers it with a random delivery delay, so messages arrive out of order on purpose.

Step 4: ingestion (ingestion.py).

- SourceClient.run calls IngestionGateway.submit(source, raw).

- parse() normalises any of the four formats to one dict: {kind:"alarm", id, reporter, entity, type, severity, state, ts, source, detail, priority}. Bad input goes to the dlq with response invalid.

- Admission: queue full gives rejected (+ retry_after); above the 60 % watermark and low priority gives rejected (shed). Otherwise the event is appended to the queue. Rejected messages are retried by the source with backoff.

- run() drains the queue at process_rate_per_s. _dedup_and_buffer drops ids already seen and pushes the event into a heap ordered by device timestamp. _release pops events older than now - reorder_window_s and appends them to the EventLog, which gives each one an offset.

Step 5: correlation (correlation.py). The supervisor's step() lets the leader engine call process(log):

- _alarm(ev): stale check (older than the last event for this entity+type, so ignore); clear handling; repeat handling; otherwise create an alarm record ALM-xxxxx, run enrich() (enrichment.py), then _correlate().

- _correlate: score against each open incident with _score() (0.6 x topology affinity + 0.25 x time proximity + 0.15 x co-occurrence). Best score >= 0.5 calls _attach; several matches call _merge; none calls _new_incident (this sets onset_ts and metrics.mttd_s).

- The first ONT_LOSi opens INC-0001; the PON_LOS, the other ONTs, PW_DOWN, E1_AIS, ABIS and SLA alarms all attach to it because they are upstream or downstream of each other in topology.yaml.

Step 6: analyze (closed_loop.py). ClosedLoopController.tick() calls step(): the incident is in phase detected; once no new alarm arrived for holddown_s (3 s) it calls analyze():

- ai.rca(active_alarms) (ai.py) scores about 26 candidates. F-A1 wins (about 89 %) because PON_LOS with laser_tx=ok implicates the feeder fibre (weight 1.0) while the port gets 0.2, fibres are a common root cause historically, and F-A1 has no healthy dependents. OLT-A loses because ONTs on PON-A2 are healthy and because OLT-A is itself reporting (so it is alive).

- topo.impacted("F-A1") returns 5 services, 2 cell sites and the subscriber count.

- _category() returns link_down (from the PON_LOS evidence).

- _match_playbooks() returns pon_type_b_switchover (fibre + link_down, and SPL-A1 is protected) and noc_investigation; ai.rank_playbooks() orders them by historical success.

Step 7: decide (decide()). Root F-A1 has policy class ODN; policy.yaml says ODN x link_down = supervised. So the phase becomes awaiting_approval and loop.pending holds the exact step (pon_protection_switch on target PON-A1). Even if you set the cell to autonomous, the blast radius (5 > 4) would escalate it.

Step 8: act. You press Approve, which sends POST /api/incidents/INC-0001/approve to operator(). act() records ACT-0001, sets phase acting and schedules _execute, which calls DeviceAdapter.execute(). That resolves the managing element (topo.manager_of gives OLT-A), checks the circuit breaker, simulates protocol latency, checks reachability, and calls sim.apply_action("pon_protection_switch", "PON-A1"), which re-points SPL-A1's depends_on from F-A1 to F-A1P. The result is queued in pending_results and applied on the next tick by _apply_result, which moves the phase to verifying with an 8 s window.

Step 9: the network heals. On the next sim.tick() SPL-A1 is up again via the standby feeder, so all those alarms are no longer expected. clear events flow through the same pipeline and the engine marks the alarms cleared. The OLT also raises PON_PROTECTION_UNAVAILABLE (warning): protection is now lost until the fibre is fixed.

Step 10: verify (verify()). Every second it checks that no service-affecting (critical, major or minor, non-AI) alarm is active, plus the playbook KPI if any. It remembers when it first became OK (first_ok_ts) and counts relapses. At the deadline it passes and calls resolve(inc, "operator", first_ok_ts): MTTR is computed, a follow-up field ticket FLD-xxxx is opened (repair the fibre), and the incident is appended to history (history.record_incident).

Step 11: the screen. Runtime.snapshot() is served by GET /api/snapshot every second and App.jsx re-renders. The entity view (Runtime.entity_view) marks F-A1 as root and its descendants as impacted, which TopologyView.jsx draws.

If you understand these 11 steps you understand the system. The other scenarios take different branches of the same code:

- E1 timing slips: decide() finds G702 x timing_sync = autonomous, so act() runs immediately and verify checks slips <= 0 on fresh telemetry.

- PON optics: autonomous re-calibration; the simulator makes the fault vanish for 2.5 s then come back; verification sees a relapse; decide(step 2) finds step risk high > guardrail medium and calls escalate().

- Rx drift: no device alarm; ai.analyze_telemetry calls _predict, which raises PREDICTED_FAULT, opening a predictive incident that is handled as advisory.

## 9. Module-by-module deep dive

### 9.1 ontology.py: the network model

- Topology.__init__ reads topology.yaml, copies class properties (layer, kind, domain, policy_class, observable) onto each entity, and builds the reverse index dependents.

- ancestors(eid) / descendants(eid): breadth-first closures with hop distances (cached; the cache is rebuilt by set_dependencies, which protection switching uses).

- affinity(a, b): correlation strength (see section 8 step 5 and docs/AI_LOGIC.md). Siblings only count when they share a "shared-risk" ancestor such as a splitter or PON port, not merely the same OLT chassis (NON_SHARED_RISK).

- impacted(eid): descendants grouped into services, devices, cell sites and a subscriber total.

- manager_of(eid): which element executes an action (ONT/E1 via its OLT over OMCI, pseudowire via the CES gateway, a link via its upstream end).

- layout(): computes x/y for the topology view (fixed rows per class, leaves spread evenly, parents centred).

- AlarmCatalog: lookups into alarm_catalog.yaml; implicates(type, detail) returns the RCA evidence weights, with when: overrides depending on alarm details (e.g. laser_tx: ok).

### 9.2 simulator.py: the fake network

- State: faults (entity to Fault(type, since, params, until)), mgmt_unreachable (devices whose management plane is down), extra_alarms (noise), active (alarms currently emitted).

- Fault types: cut, fail, power (down); optics_degraded, timing, rx_drift (degraded; rx_drift turns into down below sensitivity minus 2.5 dB).

- _recompute propagates the worst status down the dependency tree.

- _expected_alarms is the observer model: one block per entity class saying which element reports what. If the reporter itself is down, it cannot report (this is why an OLT failure produces NE_UNREACHABLE from the NMS instead of ONT alarms).

- telemetry(): per-OLT batches (OLT CPU/temp, PON BER/laser bias/utilisation, ONT Rx power/temp/BER, E1 ES/SES/slips) and a CES-gateway batch (pseudowire underruns/loss). Faults shape the numbers.

- apply_action: how each remediation changes the network. This is the only place where actions have effects.

- noise_event: customer power loss on an enterprise ONT, PDH microwave fade on E1-301/302, CPU spike on an aggregation switch.

- truth_onset: used only for KPI measurement (MTTD/MTTR), never for analysis.

### 9.3 sources.py: wire formats and senders

- render(ev) produces the native format. SNMP uses a private enterprise OID 1.3.6.1.4.1.64999 with a trap OID per alarm type (TRAP_OID). Syslog uses RFC 5424 with structured data [alarm@64999 id=... entity=... state=... severity=... seq=...].

- SourceClient: bounded buffer, random delivery delay, retry loop with exponential backoff and full jitter, and latest-value-wins for telemetry (stale samples are dropped, not retried). recent keeps sent payloads for the replay drill.

### 9.4 ingestion.py: the front door

- EventLog: an append-only list with offsets (a stand-in for a Kafka topic); keeps the last 60k events.

- IngestionGateway.parse: four parsers, one normalised output. Unknown entity or format raises ValueError and the message goes to the DLQ.

- submit: admission control. Priority 2 = critical/major or any clear; priority 1 = everything else; telemetry = 0.

- run / _dedup_and_buffer / _release: de-duplicate, re-order, append to the log; counts duplicates, reordered, late.

- snapshot(): what the resilience panel shows.

### 9.5 correlation.py: the brain's memory

- The engine's whole state is one JSON-serialisable dict (empty_state()): offset, alarms (id to record), index (entity|type to current alarm id), incidents, last_ts (per-key newest timestamp), telemetry (latest metrics per entity), stats. Keeping it plain data is what makes checkpoints and failover trivial.

- Alarm lifecycle in _alarm: stale is ignored; clear marks cleared; a repeat of an active alarm increments its count; a re-raise of a cleared alarm in an open incident reactivates it (flap); anything else is a new alarm that is enriched and correlated.

- _score ignores non-service-affecting warnings as anchors (a CPU spike must not glue unrelated faults together) and falls back to a strict rule in safe mode.

- _new_incident sets kind = reactive, anomaly or predictive.

- EngineSupervisor: start, crash (drill), step (leader work, heartbeat, checkpoint; or failover: load checkpoint, replay from its offset, promote standby), snapshot.

### 9.6 enrichment.py

enrich(ctx, alarm) returns topology (class, layer, OLT, PON port, splitter, path to core, managing element, site), device (vendor, model, software, install date, serial), impact (services, cell sites, subscribers) and history (incidents in 30 days, MTBF, last incident). It is computed once when the alarm is created.

### 9.7 ai.py: models

- train(sampler): an IsolationForest per telemetry family on healthy samples from the simulator; a LogisticRegression for prediction on synthetic windows.

- analyze_telemetry(ev): returns derived alarm events (AI_ANOMALY raise/clear, PREDICTED_FAULT raise/clear). Note the consecutive-sample logic that avoids flapping.

- observe_alarm / update_rate: alarm-storm detection (EWMA z-score).

- cooccurrence: NPMI from history.

- rca(alarms): the Bayesian scorer. Read it together with docs/AI_LOGIC.md section 3; the table of likelihoods there maps one-to-one to the if/elif chain in the code.

- rank_playbooks: success rate x specificity, with similar-incident stats.

- rule_based_rca: the safe-mode fallback.

- Every public method calls check() first, which raises AIUnavailable when up is False. Callers catch it.

### 9.8 closed_loop.py: deciding and acting

- Policy: loads policy.yaml; level(class, category); live edits via set_level / set_guardrail.

- CircuitBreaker: closed, then open after N failures, then half-open after the reset time.

- DeviceAdapter.execute: no-device actions (watch, ticket, dispatch) skip the network; device actions resolve the manager, check the breaker, retry with timeout, and call sim.apply_action.

- ClosedLoopController: tick calls step per incident (the phase machine), plus analyze, decide, act, _apply_result, verify, escalate, resolve, and operator (the human actions from the UI).

- All loop state is stored inside the incident dict (incident["loop"]), which lives in the engine state, so it survives failover.

### 9.9 history.py

SQLite with two tables: incidents (history, seeded + live) and checkpoints. seed() generates 30 days (days 1 to 15 mode=rule, 16 to 30 mode=ai) from ROOT_MIX. refresh_stats() derives priors, co-occurrence and playbook success. daily_trends() feeds the trend charts.

### 9.10 runtime.py and main.py

Runtime is the composition root: it constructs every component in dependency order, owns the background tasks, implements scenarios (SCENARIOS, run_scenario, guided_demo), drills (DRILLS, drill, _storm, _replay), KPIs (kpis), the topology status overlay (entity_view) and the dashboard payload (snapshot). main.py is a thin FastAPI layer; all routes are async so they run on the same event loop as the background tasks (no thread-safety issues).

### 9.11 Frontend

- api.js: the api() fetch helper; usePoll(path, ms) keeps the last good value and reports errors (the "link lost" banner); colour and time formatters.

- App.jsx: polls /api/snapshot every 1 s and /api/topology every 4 s; holds UI state (tab, selected incident or entity, focus); renders the header, KPIs, scenario bar and tabs.

- TopologyView.jsx: pure SVG from layout + entity_view.

- IncidentDetail.jsx: the drawer; also fetches /api/incidents/{id} for enriched alarm records.

- Panels.jsx: incident cards and list, alarm feed, event log, entity panel.

- ClosedLoopBoard.jsx, AutonomyResilience.jsx, Trends.jsx: the other tabs (Recharts for charts).

There is no client-side state store and no router; the backend snapshot is the single source of truth.

## 10. Configuration files explained

All config lives under backend/config/. Restart the backend after editing. Policy can also be edited live in the UI, but those edits are not written back to the file.

### topology.yaml

ontology:            # classes, layers, relationships (documentation + class properties used by code)
devices:             # boxes and ports:  id, class, name, depends_on, meta
links:               # fibres, E1s, uplinks, pseudowires: id, class, a, b, depends_on, attributes
services:            # end-to-end services: id, class, name, depends_on (every element on the path), meta

depends_on always points upstream. A splitter may have protection: {type, working, standby}.

### alarm_catalog.yaml

Per alarm type: category, severity, domain, ref (standard), implicates (RCA evidence weights by entity class) and optional when overrides keyed on alarm details.

### playbooks.yaml

Per playbook: match (either alarm_types, or root_classes + categories, with "*" as wildcard), steps (action, target [root | protected_port | root_pw], risk, protocol, description, optional rollback), verify (window_s, optional kpi: {metric, max|min}), and an optional follow_up.

### policy.yaml

matrix[device_class][fault_category] = advisory | supervised | autonomous and guardrails (max risk, max blast radius, min RCA confidence, rate limit, safe-mode ceiling).

### settings.yaml

| Section | Notable keys |
| --- | --- |
| simulation | tick_s, telemetry_interval_s, background_noise, trap_retransmit_ratio, network_jitter_s |
| ingestion | queue_capacity, high_watermark, process_rate_per_s, reorder_window_s, retry base/cap, syslog_udp_port |
| correlation | window_s, join_threshold, min_topology_affinity, holddown_s, heartbeat/failover/checkpoint timings |
| closed_loop | verify_window_s, action_timeout_s, action_retries, breaker settings |
| ai | anomaly_quantile, anomaly_consecutive, prediction_horizon_s, rx_sensitivity_dbm |

## 11. The API

Full interactive docs are at http://localhost:8000/docs. The important endpoints:

| Endpoint | Use |
| --- | --- |
| GET /api/health | liveness + component status |
| GET /api/snapshot | everything the UI shows |
| GET /api/topology | ontology, entities, layout |
| GET /api/incidents/{id} | one incident with enriched alarm records |
| POST /api/incidents/{id}/approve, execute, reject, ack, resolve | operator actions, optional body {"note": "..."} |
| GET /api/policy, PUT /api/policy/matrix, PUT /api/policy/guardrails, POST /api/policy/reset | autonomy |
| GET /api/scenarios, POST /api/scenarios/{name} | scenarios (guided_demo too) |
| POST /api/drills/{name} | alarm_storm, duplicates_out_of_order, crash_correlation_engine, ai_outage, device_unreachable |
| POST /api/reset with body {"full": true} | repair network (and clear incidents) |
| POST /api/noise/on, POST /api/noise/off | background noise |
| GET /api/history/trends | 30-day daily KPIs |
| POST /api/ingest/{snmp, syslog, stream, telemetry} | push external events; returns 202, 429 + Retry-After, or 400 |

Examples:

# inject a fault and look at incidents
curl -X POST localhost:8000/api/scenarios/autonomous_e1_timing
curl -s localhost:8000/api/snapshot | python3 -m json.tool | less

# push your own event (it must reference an entity id from topology.yaml)
curl -X POST localhost:8000/api/ingest/stream -H 'content-type: application/json' -d '{
  "event_id":"my-1","producer":"BSC-1","type":"CALL_DROP_RATE_HIGH","entity":"SVC-2G-201",
  "state":"raise","severity":"minor","event_time":"2026-10-20T12:00:00Z"}'

# send a real syslog datagram to the UDP listener
echo '<132>1 2026-10-20T12:00:00Z AGG-2 fmd - HIGH_CPU [alarm@64999 id="x1" entity="AGG-2" state="raise" severity="warning" seq="1"] cpu' \
  | nc -u -w1 localhost 5514

## 12. Testing and verification

backend/scripts/verify_demo.py is an end-to-end test that drives the running backend through every scenario and drill and asserts the outcome (17 checks). Run it after any change:

./run_backend.sh            # terminal 1
cd backend && .venv/bin/python scripts/verify_demo.py     # terminal 2, takes about 4-5 minutes

It turns background noise off during the run and back on at the end. Exit code 0 means all checks passed. When a check fails the script prints which wait timed out; open the dashboard to see the state it got stuck in.

Quick manual smoke test: start both servers, click each scenario button, confirm the expected phase from the README demo script, and run each drill once.

There are no unit tests yet. Good first candidates: Topology.affinity, AIService.rca on fixed alarm sets, IngestionGateway.parse per format, CircuitBreaker state transitions, and Policy guardrail decisions.

## 13. How to make common changes (recipes)

### Add a new ONT and cell site

- In topology.yaml add a drop fibre link (a: SPL-xx, b: ONT-new, depends_on: [SPL-xx]) and the ONT (depends_on: [D-new]). If it carries 2G, also add an E1Circuit, a BTS, a Pseudowire (depends_on: [E1-x, CESGW-1]) and a service (depends_on: [BTS-x, PW-x, BSC-1]).

- Restart. The layout, simulator, telemetry, RCA and impact pick it up automatically.

- If you want a fresh history that includes the new element, delete backend/data/history.db*.

### Add a new alarm type

- Add it to alarm_catalog.yaml with category, severity, ref and (if it is evidence for a root cause) implicates.

- Make something emit it: a rule in NetworkSim._expected_alarms (simulator), or send it via /api/ingest.

- If it comes by SNMP, add it to TRAP_TYPES in sources.py so it gets a trap OID.

### Add a new playbook or remediation action

- Add the playbook to playbooks.yaml with a match and steps.

- Implement the action's effect in NetworkSim.apply_action (and, for a real network, in DeviceAdapter.execute).

- If the action needs no device (ticket, workforce), add it to DeviceAdapter.NO_DEVICE.

- Optionally add history rows for it in ROOT_MIX in history.py so it gets a success rate.

### Change autonomy

Edit policy.yaml (permanent) or use the matrix in the UI (until restart).

### Add a new demo scenario

- Add an entry to SCENARIOS in runtime.py (title, story, expect).

- Add a branch in Runtime.run_scenario that injects the fault(s).

- Add a button in SCENARIO_BUTTONS in App.jsx.

- Add a check to verify_demo.py.

### Add a new telemetry metric to anomaly detection

- Emit it in NetworkSim.telemetry and in normal_samples (the training data).

- Add it to the family's feature list in FAMILIES in ai.py (and to LOG_FEATURES if it spans orders of magnitude).

### Connect a real data source (first step toward production)

- Syslog: point devices at UDP 5514 using the structured-data format shown in section 11, or adapt SYSLOG_RE / IngestionGateway.parse to your vendor's format.

- SNMP: run a trap receiver (e.g. pysnmp or snmptrapd) that converts traps to the dict shape in sources.render and POSTs them to /api/ingest/snmp, mapping your vendor's MIB OIDs in OID_TRAP.

- Event streams: a Kafka consumer that POSTs to /api/ingest/stream.

- Disable the simulator's emissions (stop _sim_loop / _telemetry_loop in Runtime.start) and replace the NetworkSim.apply_action calls in DeviceAdapter with real NETCONF/OMCI/TL1 clients.

## 14. Debugging and troubleshooting

| Symptom | Likely cause and fix |
| --- | --- |
| Dashboard shows "Backend not reachable" | Backend not running or not on :8000. Check terminal 1; run curl localhost:8000/api/health. |
| Address already in use | Another process is on 8000 or 5173. Stop it, or run PORT=8001 ./run_backend.sh and set VITE_BACKEND=http://localhost:8001 before npm run dev. |
| "UDP syslog listener disabled" in the activity log | Port 5514 is busy. Harmless, or change syslog_udp_port (0 disables). |
| pip install fails | Needs Python 3.10+ and internet or proxy access; delete backend/.venv and retry. |
| Nothing happens after clicking a scenario | Wait about 4 s (hold-down). Check System activity. Run Repair network if a previous fault is still active. |
| Incident stuck in a phase | Open the drawer: the timeline and decision explain why (e.g. awaiting approval). |
| Prediction did not appear | It needs 16 to 40 s of telemetry; the drift scenario must be running and the AI must be online. |
| Odd correlations | Tick show simulator ground truth; background noise can coincide with your scenario, so switch noise off. |
| Want to see raw events | GET /api/snapshot and look at ingestion and engine_stats, or add a print() in IngestionGateway.parse. Uvicorn logs go to terminal 1. |

Debugging tips:

- The whole engine state is one dict: rt.supervisor.leader.state. In a debugger or a temporary endpoint you can inspect any alarm or incident directly.

- Every phase change writes to incident["timeline"]; read that first.

- Timings are in settings.yaml; slowing tick_s or raising holddown_s makes behaviour easier to watch.

## 15. Known limits and the path to production

| Limit (PoC) | Production approach |
| --- | --- |
| Simulated network and actions | Real collectors and southbound clients (NETCONF/RESTCONF, OMCI via OLT EMS, TL1), ticketing and workforce APIs |
| SNMP traps arrive pre-decoded | Trap receiver with vendor MIBs feeding /api/ingest/snmp |
| One process, in-memory event log | Separate services; Kafka for the log; leader election via Kubernetes lease or etcd |
| SQLite history and checkpoints | PostgreSQL/TimescaleDB; object storage for checkpoints |
| Synthetic 30-day history | Train on the operator's real alarm, incident and ticket history; periodic retraining and model monitoring |
| No authentication, single user | SSO, role-based approvals (who may approve which risk level), audit log of every automated action |
| UI polls every second | WebSocket/SSE push for large deployments |
| Policy edits in the UI are not persisted | Store policy in a database with change history and approvals |
| No unit tests, one E2E script | Unit tests per module, contract tests for parsers, chaos tests for fault tolerance |

## 16. Glossary

| Term | Meaning |
| --- | --- |
| Abis | Interface between a 2G BTS and the BSC |
| ACR | Adaptive clock recovery, how a CES port derives timing |
| AIS | Alarm Indication Signal; "upstream failure" filler on E1 |
| BER | Bit error rate |
| BSC | Base Station Controller (2G) |
| BTS | Base Transceiver Station (2G cell site) |
| CES / CESoPSN | Circuit emulation (carrying E1 over packets) / the RFC 5086 method |
| Circuit breaker | Stops calling a failing dependency for a while |
| dBm | Optical power in decibels relative to 1 mW |
| DLQ | Dead-letter queue for unparseable messages |
| E1 / E3 | 2048 kbit/s / 34368 kbit/s TDM rates (G.702) |
| EWMA | Exponentially weighted moving average |
| G.702 / G.703 / G.704 | ITU-T standards for bit-rate hierarchy / electrical interface / framing |
| G.775 / G.822 / G.826 | ITU-T standards for LOS/AIS criteria / slip objectives / error performance |
| G.984 | ITU-T GPON standard |
| IsolationForest | Unsupervised anomaly-detection algorithm |
| LOS / LOF | Loss of signal / loss of frame |
| MTBF | Mean time between failures |
| MTTD / MTTR | Mean time to detect / resolve |
| NETCONF | Network configuration protocol (RFC 6241) |
| NMS / EMS | Network / element management system |
| NOC | Network Operations Centre |
| NPMI | Normalised pointwise mutual information (co-occurrence strength) |
| ODN | Optical distribution network (passive fibre plant) |
| OLT / ONT | Optical line terminal / optical network terminal |
| OMCI | ONT management and control interface (the OLT manages ONTs through it) |
| OTDR | Optical time-domain reflectometer; locates fibre breaks |
| PDH | Plesiochronous digital hierarchy (E1/E2/E3 multiplexing) |
| PON | Passive optical network |
| PW | Pseudowire |
| RCA | Root-cause analysis |
| SA | Service assurance system |
| SDH / STM-1 | Synchronous digital hierarchy / its 155.52 Mbit/s level |
| SFP | Pluggable optical transceiver |
| Slip | A dropped or repeated frame caused by clock mismatch |
| TL1 | Text-based management language used by TDM equipment |
| Type-B protection | GPON protection with duplicated feeder fibre and OLT ports |

## 17. Your first week: suggested exercises

- ☐ Day 1: run the system, do the five-minute demo from the README, and read sections 1 to 5 of this guide.

- ☐ Day 2: follow section 8 with the code open; put a breakpoint or print in CorrelationEngine._new_incident and in ClosedLoopController.decide and watch a scenario hit them.

- ☐ Day 3: change ODN x link_down to autonomous in the UI and run the fibre cut. Explain why it still asks for approval (blast radius guardrail). Then raise max_blast_radius_autonomous to 10 and run it again.

- ☐ Day 4: add a fifth ONT under SPL-B1 with an enterprise service (recipe in section 13), restart, cut its drop fibre via a new scenario, and check that the RCA names the drop fibre.

- ☐ Day 5: run verify_demo.py; then write your first unit test for Topology.affinity and one for AIService.rca using the cascade alarm set.

Further reading inside the repo: docs/ARCHITECTURE.md, docs/AI_LOGIC.md, docs/CLOSED_LOOP.md, docs/FAULT_TOLERANCE.md, docs/METRICS.md, docs/TOPOLOGY_ONTOLOGY.md.